# EZGUIDETM Fixed Guide Rail Owner’s Manual

Before installing or operating EZGUIDETM Fixed Guide Rail system, read and understand this Manual.

Failure to follow instructions and safety precautions could result in serious injury, death, or property damage.

Table of Contents

Safety     ……………………………………………………………………………………………………………..	4

General Information     …..…………………………………………………………………………………..	5

Overview     …………………………………………………………………………………………………………………………………..	5

Guide Rails     ……………………………………………………………………………………………………………………………….	              6

Extended Distances     …………………………………………………………………………………………………………………..	              6

Bracket Installation Guide     ……………………………………………………………………………….           7

Straight Conveyors     ………………………………………………………………………………………………………………                          7

Curved Conveyors     …………………………………………………………………………………………………………………….                8

Bending Guide Rail     …..……………………………………………………………………………………..         9

Introduction     ………………………………………………………………………………………………………………………………            9

Preparation     ……………………………………………………………………………………………………………………………….           10

EZGUIDETM Hand Bender Instructions      ……………………………………………………………………………………….           11

EZGUIDETM Rail Roller Instructions     ………………………………………………………………………………………              12-14

Guide Rail Installation     …………………………………………………………………………………….    15-17

Vertical Bends                   …………………………………………………………………………………….          18

Installation

When installing EZGUIDETM Fixed Guide Rail, ensure the guide rail clears the top of the conveyor before operation. Inadequate clearance can cause damage to both the conveyor and EZGUIDETM.

Operation

Never adjust the EZGUIDETM Guide Rail while the conveyor is in motion. Follow your Standard Operating Procedure and Safety Protocols to shut down the conveyor before adjusting the Guide Rail to prevent damage or personal injury.

Prohibited Environments

EZGUIDETM Fixed Guide Rail should not be used in certain environments. If you are unsure of the safety or suitability of your intended environment, contact a qualified Span Tech representative. Never use EZGUIDETM Fixed Guide Rail conveyor in any of the following environments:

• Where chemicals that react with acetal copolymer are used

• Where strong acids or caustics are present

• Where ultraviolet light is present

• Where flammable materials are present (i.e. gasoline, solvents, etc.)

In Case of Fire

The acetal material of the conveyor chain burns with a very hot, very faint flame. In case of fire, use Water, Foam, CO2, or Dry Chemical extinguishers only. Use of other material will not extinguish the fire and could result in serious injury, death, or significant property damage.

If an EZGUIDETM Fixed Guide Rail acetal bracket were to catch fire, a blue flame would be barely visible, and little or no smoke would be produced. In case of fire, immediately stop conveyor system operation. The fire can then be extinguished using a water, foam, CO2, or dry chemical fire extinguisher. Report any fire to the applicable Fire Department and plant management personnel immediately. Do not re-operate conveyor until all repairs have been made.

# General Information

## Overview

EZGUIDETM Fixed Rail is a non-adjustable guidance system designed for product retention on a conveyor. The EZGUIDETM Fixed Rail is designed for both straight and curved conveyors. Sheet metal and molded channel bracket options are available, allowing for multiple conveyor offsets. EZGUIDETM Fixed Rail can act as a stand-alone guide rail system or be paired with the other EZGUIDETM configurations to allow for adjustability.

## Guide Rails

The rails used in the EZGUIDETM system are 45mm (1.77in) tall and have (2) profiles:

- Flat Face Anodized Guide Rail is a military grade hardcoat anodized aluminum rail with a 30mm flat contact surface.

- Flat Face Coated Guide Rail is a white HDPE (high density polyethylene) coated aluminum rail.

## Extended Distances

To extend a Fixed Rail system beyond the length of a single rail, simply assemble (2) Guide Rails together end to end and use a Guide Rail Field Break to hold the (2) rails together.

# Bracket Installation Guide

## Straight Conveyers

For a straight conveyor (no curves), follow the directions in this section.

- Place brackets as close to the end of the conveyor as possible.

- To ensure the rigidity of the guide rail the Maximum Overhang is 347mm (13.6in).

- Place the EZGUIDETM Fixed Guide Rail Brackets along the conveyor between the end brackets using a Maximum Spacing between bracket assemblies of 610mm (24in).

- Guiderail sections must maintain at least two points of contact - i.e., must be supported by two clamps.

## Curved Conveyors

- Place a bracket at the apex (midpoint) of the curve.

  - Space other brackets evenly through the curve without exceeding the Maximum Spacing of 610mm (24in) between brackets.

- There must be a minimum of (1) bracket on the straight section after the Tangent Line to keep the curved rail ends in place.

# Bending Guide Rail

## Introduction

There are (2) tool options when it comes to bending EZGUIDETM Guide Rail, the EZGUIDETM Hand Bender and the EZGUIDETM Rail Roller.

- The Hand Bender is designed to be held with a vice while bending the guide rail by hand. Due to the potential to scuff the HDPE coating, the Hand Bender is only designed to bend anodized versions of EZGUIDETM Guide Rail.

- The EZGUIDETM Rail Roller is a hand cranked roller tool designed to roll all versions of EZGUIDETM rail. The Rail Roller has several sets of rollers designed to roll the rail.

EZGUIDETM Hand Bender  				EZGUIDETM Rail Roller

## Preparation

### Step 1

Measure along the Inside and Outside Curve of the conveyor, between the Infeed Tangent and the Outfeed Tangent of each bracket to find the Edge-to-Edge Measurement.

### Step 2

Add 100mm (4in) to the Edge-to-Edge Measurement to get the Total Rail Cut Length.

## EZGUIDETM Hand Bender Instructions

### Step 1

Mount the Hand Bender in a vice and slide the rail through the Hand Bender to the marks made for the Infeed or Outfeed Tangent Length.

### Step 2

Make a series of small bends in the rail in the section between the Infeed and Outfeed Tangent Lengths.

Do not over bend the rail!  It is better to “under bend” until reaching the desired radius.

## EZGUIDETM Rail Roller Instructions

### Step 1

When using the EZGUIDETM Rail Roller it is important to select the correct set of rollers for the guide rail being used.

- Each roller set is shaped to a specific guide rail profile.

- Guide rail roller sets contain (4) rollers, (2) white rollers and (2) black rollers.

- Idler Rollers w/ Bearings and Drive Rollers (w/ Keyway).

### Step 2

Place the rollers on the device in the intended configuration and tighten them down using the hand knobs.

- The Rail Roller has (2) configurations; one for rolling inside curves and one for rolling outside curves.

- The inside curve configuration uses the white idler roller (with bearings) and has the black drive roller (keyed) placed on top.

- The outside curve configuration has the white drive roller (keyed) on top and uses the black idler roller (with bearings).

### Step 3

Insert the guide rail between the rollers in the correct orientation to cradle the rail by the rollers. Position the rail so the marked tangent point is centered with the top roller.

### Step 4

Spin the knurled adjustment knob to bend the rail at the marked tangent point. To decrease the possibility of over-bending, use a series of small adjustments to get the desired angle.

### Step 5

Turn the crank handle to roll the guide rail between the tangent marks.

# Guide Rail Installation

- Measure and Cut Guide Rail

Measure and cut the straight pieces of guide rail for the system. Refer to the Bending Guide Rail - Preparation section to get the curved sections of guide rail.

- Loosen Rail Clamps

To install the guide rail into the Fixed Guide Rail brackets, use a 2mm hex wrench to loosen the screw in the center of the guide rail clamp of each bracket.

- Insert Guide Rail

  - Mounting in Straight Conveyor Sections

Slide the guide rail into the clamp at the end of the conveyor and repeat the process for all the brackets in the straight sections of the system.

  - Mounting in Curved Conveyor Sections

Start with the center bracket and work outwards. It may be necessary to loosen the (2) clamp halves and attach the clamp over the guide rail as opposed to sliding it through the clamp.

- Overlap the Guide Rail in the Direction of Flow

At locations where the guide rail will overlap refer to the direction of flow below. Overlap straight pieces of guide rail 25mm (1in) past the clamp.

- Position Guide Rail

With the guide rail installed in the system, make any positioning adjustments by sliding the guide rail through the clamps while they are all loose.

- Tighten Rail Clamps

Once the guide rail is positioned properly, finish tightening the clamp screws on any standard bracket clamp to lock the rail in place. Tighten the center curve bracket clamp to lock the curve rail in place. The remaining curve clamps must allow the rail to slide through the clamp for radius changes.

- Height Adjustments

Height adjustments to the system can be made by sliding the clamp in the guide rail bracket slot.

When making height adjustments to the guide rail, provide clearance between the bottom of the guide rail bracket and the top of the chain.

- The EZGUIDE Vertical Bend Assembly allows for angles ranging from (0 to 40 Deg.)

  - Mount a Guide Rail Bracket on each side to support the Vertical Bend.